ERDMS (ERI) Reference Data Description

(Overview)

Contents

1	Introduction	1

1.1	Revision overview	1

1.2	References	2

2	Description of ERI reference data in ERDMS	3

2.1	Hazardous substances / goods (ADN(R) from the CCR)	4

2.2	Non-hazardous substances (HS-code)	6

2.3	Locations, terminals (based on UN Rec 16 and RIS index)	8

2.4	Countries and country codes (ISO 3166-1)	12

2.5	Ship type (UN Rec 28)	13

2.6	Container type (ISO 6346)	15

2.7	Inner Packaging type (UN Rec 21)	25

Bijlage A -  Data maintenance and tooling	26

Bijlage B -  Reference Data update procedure (overview)	27

# Introduction

This document describes the set of reference data as used in Electronic Reporting International (ERI) by the various national and international shipping systems (i.e. BICS, ERI-Net, MIB-II, IVS90 etc.).

Reference data refers to the codes and descriptions (code lists) as used in the different EDI messages to exchange information about voyages and cargo by the several national and international shipping systems. This includes, for example, location and terminal codes, ship types, container types and hazardous and non-hazardous substances codes.

Especially in the context of electronic reporting the usage of standardised messages and codes is very important. That’s why these code lists are maintained on a central European level and published from a central location/ database using the so called European Reference Data Management Service (ERDMS) tool.

The purpose of this document is to give a global overview, including a short description (background information) about the current situation of the Reference data as available in the ERDMS. Also some practical guidelines are given about maintaining and encoding the data as used at the moment in the different ERI shipping applications and systems. It is expected from the reader that he has some knowledge about the RIS and especially the ERI context.

The rest of this document describes the different code lists used in ERI and available in ERDMS at the moment. Per code list the following items are described: a short description about the data, usage in ERI, the key elements, sources and International standards used, including some basic (practical) encoding guidelines for defining new codes.

## Revision overview

| Version | Date | Description |
| --- | --- | --- |
| V0.1 | 1 April 2010 | Initial summary for RefTool |
| V1.0 | 18 July 2011 | Update for ERDMS |
| V1.01 | 3 Nov 2011 | Update |
| V1.02 | 22 Oct 2012 | Update (HS codes) |
| V1.04 | 01 Oct 2013 | Update (ADN2013) |
| V1.05 | 21 July 2015 | Update (ISO6346-1995 Amd 3 2012) |

The footer of each page gives the document number, the revision and the version number of the document.

## References

This document is based on a number of documents that were written previously. Where necessary a reference indicated by [] will be used to refer to the contents of these documents.

The reference documents (official standards) are:

- Regulation 164/2010, technical specifications for electronic reporting on inland waters. dated 25 January 2010 (supersedes ERI Guide 2.0).

- ERI Guide 2.0, Part IV Codes and References, dated august 2006.

- Platina_annex 3_Maintenance procedure ref data.doc, dated 22-08-2010.

- Platina consolidation RIS index and ref data.doc, dated 22-08-2010.

- RIS index encoding guide v2.0 dated 2015-06-23.

# Description of ERI reference data in ERDMS

The following ERI reference data is available in ERDMS:

- Hazardous substances / goods (ADN(R)

- Non-hazardous substances / goods (HS)

- Locations (berths) and terminals

- Country codes

- Ship types (hulls and combinations)

- Container type

- (Inner) Packaging type

All the above mentioned reference data as used in Electronic Reporting application are described in detail in separate documents and standards (see ref [1] and [2] annex IV).

The actual reference data (also see footnote Error: Reference source not found) is available in the so called EU Reference Data management Service, Tool (ERDMS) and can be downloaded from this Tool .

See http://risdatamanagement.ris.eu (use the Masterdump option)

This ERDMS Tool can also be used to request mutations and maintain the different codes (account needed).

The following sections give a short description of the various types of the ERI reference data and some basic (practical) encoding guidelines as they are applied at the moment on the data in ERDMS. Especially the data (codes) used in Electronic Reporting International (ERI) by the different ERI applications.

It speaks for itself that the encoding and generation of the codes will have to follow the applicable standards and procedures (see ref [3]). The encoding guidelines in this document are intended only as a practical guide in addition to the corresponding Int. Standards and User manuals of the ERDMS Tooling.

## Hazardous substances / goods (ADN(R) from the CCR)

### Description ADN data

Figure 1: ADN codes overview

For hazardous substances the International ADN(R) regulation is used. ADN(R) stands for: Accord européen relatif au transport international des marchandises Dangereuses par voie de Navigation intérieure (du Rhin). Nowadays shortened to ADN.

This regulation from the Central Commission for the Navigation on the Rhine (CCR) and published by the UN sets down for a list of dangerous substances which can be transported in cargo vessels and/or tankers, the identification numbers (UN no.), classifications, names, synonyms/details and additional information (cone calculation parameters, provided by specialist of the Dutch Rijkswaterstaat).

The UN no, class & classification, packing group and proper shipping name are used in Electronic Reporting to identify the dangerous substance. The additional cone calculation parameters are used by some application to calculate the number of cones.

### Standards used

The ADN agreement, the European Agreement concerning the International Carriage of Dangerous Goods by Inland Waterways (ADN 2013) from the Central Commission for the Navigation on the Rhine (CCR) published by the UN.

See also: 	http://www.ccr-zkr.org (document, download section) or the UNECE website: 	http://www.unece.org/trans/danger/danger.html (ADN section).

More information:

- ADN (UNECE web site Transport / Dangerous good section) :

http://www.unece.org/trans/danger/publi/adn/adn_e.html

- ADN codes and regulation (UNECE):

http://www.unece.org/trans/danger/publi/adn/adn2013/13files_e.html

http://www.unece.org/trans/danger/publi/adn/adn2015/15files_e.html

### Encoding guideline ADN data

The internal (unique) 7 digit ADN code is constructed using the 4 digit UN number and by adding a 3 digit sequence number (starting with 000) for each same UN number entry (sequences remain with that entry and are not reused after deletion).

In basic a ADN substance can be identified on UN number, Class, Classification and or Packing group. This may not return a unique substance (same substance identifications but with different names), but these substances will have the same cone calculations parameters).

The ADN codes, additional information and parameters are all based on the UN publications of the ADN regulation (especially Table A and C).

The ADN Name field must contain the Proper Shipping name from ADN and the Synonym (detail) field will contain the additional details such as flashpoint, vapour pressure etc. In basic the Proper shipping name is based on the uppercase text from the ADN tables (Column 2) and the additional details are based on the lowercase (there are exceptions for the Table C substances).

### Examples ADN data

The 7-digit UN serial number (a combination of UN no. and internal 3-digit serial number) is the unique key to the Hazardous substances table.

Example:

| ADN Serial no. | UN no | Class | Classif | Pack. Group | NAME |
| --- | --- | --- | --- | --- | --- |
| 1202001 | 1202 | 3 | F1 | III | GAS OIL or DIESEL FUEL or HEATING OIL, LIGHT (61 C) |
| 1203001 | 1203 | 3 | F1 | II | MOTOR SPIRIT or GASOLINE or PETROL |
| 1204001 | 1204 | 3 | D | II | NITROGLYCERIN SOLUTION IN ALCOHOL |
| 3316000 | 3316 | 9 | M11 | II | CHEMICAL KIT OR FIRST AID KIT |
| 3316001 | 3316 | 9 | M11 | III | CHEMICAL KIT OR FIRST AID KIT |
| 3412000 | 3412 | 8 | C3 | II | FORMIC ACID (with 10% - 85% acid by mass) |
| 3412001 | 3412 | 8 | C3 | III | FORMIC ACID (with 5% - 10% acid by mass) |

## Non-hazardous substances (HS-code)

### Description HS-code

Figure 2: HS-Code overview

For the non-hazardous substances in ERI the International Harmonized System (HS) codes standard from World Customs is used (Chapters 01-97). From this list a subset was selected (no dangerous, no chemicals etc.), supplemented with ERI specific items (chapter 99 or temporary codes in chapter 00).

Within the ERI context HS codes up to level 6 are used (first 6 digits out of 10 are significant, there are no deeper levels in ERI, last 4 digits are used as a possible sequence number).

The 6-digit (ERI) HS code has 4 zeros added to a length of 10 digits if it is a “genuine” (official) HS code (extracted from the official WCO Customs list, chapter 01-97).

If it is a synonym or other added item (unofficial HS code) the last 4 digits are used as a (unique) serial/sequence number not equal to “0000” (e.g. 0001 etc.) => End 2011 the ERI expert group decided that only official HS (WCO) codes are allowed / will be supported, so in Q1 2012 all the unofficial HS codes in chapters 01-97 were removed from the ERI list.

So when mapping an ERI HS-code on the official Customs list only the first 6 digits of the ERI HS-code list are relevant and will match with an official Customs code.

In addition, the corresponding link with NST 2007 coding for statistics and statistical purposes has been included per HS code (initially only the first 2 digits of NST 2007 are filled in).

### Standard used

Harmonized Commodity Description and (HS) Coding System from World Customs Organization is used as the basis for the ERI HS codes.

The linked NST 2007 (2 digit) codes are based on the COMMISSION REGULATION (EC) No 1304/2007 dated 7 November 2007.

See also:

http://www.wcoomd.org (World Customs Organisation, Nomenclature section)

http://www.foreign-trade.com/reference/hscode.htm

More information:

- WCO codes: www.wcoomd.org/home_hsnomenclaturetable2012.htm

- NST 2007: www.unece.org/trans/main/wp6/transstatwp6nst.html

### Encoding guideline HS-code

When adding new entries: search for a match with an existing HS code (in the Customs list) or if an exact match is not available then search for the correct chapter/subchapter (01-97) and try to find a match in the “Other” section in that chapter, where the new HS-code can be inserted (only official HS codes ending on “0000” are allowed).

Use the first 6 digits of the corresponding (matching) chapter and subchapter for the new ERI HS-code. Add “0000” to the new code, resulting in a unique 10 digit ERI HS code.

Finally codes in chapter 00 are not based on the standard (don’t use chapter 00), existing records in this chapter will be phased out, deleted in the future. At the moment these chapter 00 codes are only available for backward compatibility reasons.

Codes which cannot be inserted in an existing chapter (01-97) must be inserted in chapter 99. Chapter 99 is available for other (specific) authority HS codes. In this case (chapter 99) the first 6 digits of the HS code to be inserted must match a subchapter in chapter 99 and the last 4 digits are used as a (unique) serial/sequence number (e.g. 0000, 0001 etc.).

### Data examples HS-code

The 10-digit HS Code is the unique key for the Non-Hazardous substances table. Only the first 6 digits (3 chapters) are significant and relevant when mapping it on the Customs HS code list.

For example:

| HSCODE | NST2007 | NAME |
| --- | --- | --- |
| 0000000000 | 20 | *CARGO NOT OTHERWISE SPECIFIED |
| 0100000000 | 01 | LIVE ANIMALS |
| 7501200000 | 10 | NICKEL OXIDE |
| 9706000000 | 20 | ANTIQUITIES >100 YEARS OLD |
| 9900009999 | 19 | UNKNOWN |
| 9909000001 | 16 | EMPTY SHIP (NO CARGO) |

## Locations, terminals (based on UN Rec 16 and RIS index)

### Description ERI Locations

Figure 3: ERI Location code overview

The ERI locations table contains all the relevant (International, worldwide) locations (harbours, berths, terminals, loading location, locks and other important locations etc.) as used in ERI (Electronics Reporting International) messages (in the future these location will be/must be integrated into the RIS-index).

The ERI locations are now based on (super set of) the UN/Locodes and the so called RIS index (see ref [4] and [5]).

The UN/Locode system is an international code system for locations throughout the world, based on the UN recommendation 16. The UN/Locode contains codes for cities, ports, airports etc. For ERI the UN/Locdes with function=1=ports are important and used.

The RIS index is a collection of important RIS objects on and along the waterways as defined per country in the context of RIS. The RIS index is also based on the UN/Locode and additional sub codes and available in countries participating in RIS.

Each ERI Location is related to a certain reference point (= the RIS index) and the relation is used to construct the ERI location code (20 chars). This may be a 1 on 1 relation (berths in the RIS index). So the ERI location code is the same and extracted from the RIS index. Or a 1 on N relation is possible where several ERI locations can relate to one RIS index (several locations linked to one nearby RIS index, usually a junction, i.e. berths in a harbour area or a harbour area or basin). In this case the ERI location code is coded using the fairway code and hectometre code of the related RIS index.

So in certain area’s (especially in harbours) several terminals can be available (must be coded). For such situations the ERI location code (UN/Locode) is extended with an additional sub code (usually from the related RIS-index), such as a waterway code, additional sub-location (terminal) code (preferable the existing berth code) and a hectometre code. The terminal and or the hectometre code must be defined in such a way that the entry will be unique.

The additional sub code (maximum 15 letters) is constructed differently per country. The general rule here is that the first part of exactly 5 letters contains the coding of the part of the fairway or harbour basin (fairway code) and the 2nd part (maximum 5 letters) may contain the mooring or terminal code (if necessary up to the number of the bollard). Finally the last part contains a fairway section hectometre of 5 digits. Preferable the additional fairway and hectometre codes are based on a nearby / related RIS index (junction, harbour area or basin).

Together the Locode (5 letters) and the sub code (0-15 letters) form a unique location code of 5 or 20 letters long. When the ERI location code (also called SRS code) has been established for a location (or the nearest code for that location), this will no longer change, unless the Locode changes. In other words, the location code is permanent; only the attributes for that location, such as the name of the terminal, terminal code etc. can change.

Definition of an ERI location code:

UN locode (5 chars) for (International) locations

or

UN locode + fairway code + terminal code + hectometre code.

(for areas where there are more sub locations / berths / terminals).

To ensure compatibility with other systems in ERI, all (basic = 5 digits) UN Locode (locodes with function=1=ports) have been included in the ERI locations (for example NLRTM=ROTTERDAM, USNYC=NEW YORK).

### Standard used

The 5-letter UN Locode is based on UN/ECE Trade Facilitation Recommendation 16 and the 2-letter ISO 3166 country code is followed by a unique 3-letter code for the location within that country.

The fairway code is country specific and depends on the definition of the fairway network in that country.

The terminal code is location specific (for a port area, preferable the berth code from the port authority).

See also:

http://www.unece.org/locode (UN locode website)

http://www.ris.eu/ (RIS website).

More information:

- UN Locodes: www.unece.org/cefact/locode/welcome.html

- RIS Index: www.ris.eu/library/expert_group/nts/guides

- BTB Container Terminals: http://www.binnenvaart.org/?page_id=1446

### Encoding guideline ERI location

Preferable the ERI location code must be based (1 on 1) on an existing RIS index items (function=berth, hrbare etc). In this case the ERI location code will be the same as the RIS index. If no (exact) corresponding RIS index is available (locations within port areas or along the fairway etc) the ERI location must be based on the nearest (related) RIS index. Parts of the related (nearest) RIS index must be used to encode the new ERI location.

The related (nearby) RIS index used for determining the fairway code etc, must be one of the following RIS index types:

- a RIS index berth object (ERI location code is the same as the RIS index code, exact match).

- a RIS index junction or harbour area or basin (ERI location code, fairway code etc is based on parts of the RIS index).

(do not relate ERI locations to other RIS index types, then the one mentioned).

ERI Location code (encoding):

Position

1-5:		UN-Locode of the nearest location with a UN-Locode (can be different then the UN-Locode as used in the RIS index.

6-10	Fairway code: the fairway code of the nearest RIS index (junction etc) on the same waterway.

11-15	Terminal code: preferable the berth code from the Port, commercial (container & tank) terminal or the object code from the RIS index.

16-20	Hectometre code: preferable the related (calculated) hectometre on the fairway or the hectometre from the related RIS index. The hectometre code = fairway kilometre setting x 10.

The ERI Location code must be unique. To make a code unique one could vary the terminal code or the hectometre code.

Unknown UN-Locodes are coded with “XXX”, such as NLXXX, DEXXX, FRXXX. However the usage of these unknown XXX codes must be minimised!

Unknown or shorter sub codes are supplemented (prefixed) with “0” (so “ABC” becomes “00ABC” or unknown = “00000”).

The Location name in combination with the Terminal name (if available) must be unique, because this is the item the user will see in ERI applications when selecting locations.

For the basic UN-Locodes the code is usually put in brackets after the location name, i.e. NLAMS=AMSTERDAM (NLAMS), so it can be recognised and to make the name unique.

For every ERI location a basic UN-locode must be available in the list to support the possibility of the so called fallback mechanism. In some cases this will result in duplications (names) and these locations must be marked as a “basic duplication code” (term type = 64).

The type of a location must be encoded as (combinations are also possible):

0 = No info

1 = Berth(s) on a fairway / in a harbour

2 = Locks, bridges and or reporting points (for routing, route points, key locks etc)

4 = Sea estuary or (seaport) harbour location

8 = Hinterland (logistic location near a fairway)

16 = Hinterland (logistic location NOT near a fairway)

32 = Fairway (not used yet)

64 = Basic duplicated location code (available for the so called fallback mechanism).

For berths in a harbour area of terminals there is the distinction between container terminals and other terminals. For these terminal / berth locations the terminal type and name needs to be filled in also (combinations are also possible).

Terminal types:

0 = No info.

1 = Container terminal.

2 = Tank terminal (liquid cargo).

3 = Combined container & tank terminal (containers and or liquid cargo).

4 = Dry cargo terminal (bulk & general cargo).

5 = Combined container & dry cargo terminal.

7 = Combined container, tank & dry cargo terminal.

10 = Control lock (route point)

11 = Control and key lock

12 = Key Lock

Some encoding examples of Locations and their types:

| Location | Loc Type | Term Type |
| --- | --- | --- |
| UN-Locode (Basic code), function 1 | 8 | 0 |
| UN-Locode (Basic code), function <> 1 | 16 | 0 |
| Berths in ports or (container, tank etc) terminals | 1 | Any > 0 |
| Port area (sea or inland) | 4, 1 | 0 |
| Locks, bridges other (ERI) passage/route points | 2 | 10, 11, 12 |
| UN-Locode (duplication code) NLAMS = Amsterdam => this is the basic duplication code. NLAMS00100J000100010 = Amsterdam | 64 1 | 0 0 |

For the Lat Long coordinates preferable the actual coordinates of the object must be used or a coordinate on the waterway nearby (use decimal Lat, Longs). If unknown 0,0 must be filled in.

The IVS reporting system is the name of a authority system related to that location, such as:

“IVS90”: The Dutch fairway monitoring system.

“MIB”: 	The German fairway monitoring system.

“VNF”: The French system.

The flag “IVS report” must be true for locations other than NL (this flag implies that a reporting system (point) has to be chosen/specified by the reporting application when departing from that location).

### Data example ERI location

The Locode and the subcode (5 or 20 letters location code) together form the unique key (location code) in the location table and are initially determined as described above, after which this code will no longer change.

Example:

| ERI Location code |  |  | 1 | 1 | 2 | 3 | 4 | UN | Lo-code | Fair-way code | Term code | Hectometer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Place of departure/ destination |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Germany; Mainz; Rhine; Frankenbach; ; | x | x | x | x |  | DE | MAI | 03901 | 00FRB | 00000 |
| 2 |  | The Netherlands; Rotterdam; Section 2552 (Oude Maas); Leuvehaven; ; | x | x | x | x |  | NL | RTM | 02552 | LEUVE | 00000 |
| 3 |  | The Netherlands; ;Section 2552 (Oude Maas); ; km 2,2 | x |  | x |  | x | NL | XXX | 05552 | 00000 | 00022 |
| 4 |  | Germany; ; Rhine; ; km 502.3 | x |  | x |  | x | DE | XXX | 03900 | 00000 | 05023 |
| 5 |  | Amsterdam (NLAMS) (basic UN locode) | x | x |  |  |  | NL | AMS |  |  |  |

Comment: The main sources of the various parts are:

- UN/Locode is based on UN/ECE Trade Facilitation Rec 16.

- Fairway code and Fairway section hectometre based on the codes from the fairway authority (see RIS index). Fwcode is left padded with zero’s if necessary.

- Term code based on the codes for berths from the Port authority, commercial terminal codes or object codes from the RIS index (left padded with zero’s).

- Hectometre based on the real situation (calculated) or the same as the corresponding RIS index.

## Countries and country codes (ISO 3166-1)

### Description Country code

The country table lists the countries of the world. The following codes have been included for each country:

- The 2-letter ISO 3166-1 UN country code.

- The 3 letter ISO 3166-1 country code.

- The 3 digit ISO 3166-1 country code.

- The 3 digit CBS country code (backwards compatibility, will be deleted).

- The 3-letter Lloyds flag code (XXX if unknown).

### Standard used

For country codes the International ISO 3166-1 standard is used. ISO 3166 is the International Standard for country codes as used in the World. The purpose of ISO 3166 is to establish codes for the representation of names of countries, territories or areas of geographical interest, and their subdivisions.

See also:

http://www.iso.org

http://www.iso.org/iso/country_codes

http://www.unece.org/locode

More information:

- ISO codes: www.iso.org

- Country ISO 3166 codes: www.iso.org/iso/country_codes

- Country names UN Rec 3: www.unece.org/cefact/recommendations/rec_index.html

### Encoding guideline ERI Country code

See ISO 3166-1 standard for the ISO codes.

The Lloydsflag codes are (additional) flag codes published by Lloyds register for ocean going vessels and are used to identify the registration flag (country) for a vessel (use XXX if unknown).

The “CBS code” is based on country codes published by the CBS statistics department in the Netherlands (not in use anymore). This “CBS code” is available for backward compatibility only and will become obsolete/will be deleted in the future.

### Data example, Country code

The universal ISO 3166-1 UN code (2 digits) is used as the key field for this code list.

Example:

| UN code | UN 3 code | UN nr | CBS code | LloydsFlagcode | Country |
| --- | --- | --- | --- | --- | --- |
| FR | FRA | 250 | 001 | FRA | FRANCE |
| BE | BEL | 056 | 002 | BLG | BELGIUM |
| NL | NLD | 528 | 003 | NTH | NETHERLANDS |
| DE | DEU | 276 | 004 | GEU | GERMANY |

## Ship type (UN Rec 28)

### Description Ship type

Ship type codes are used to identify the type of ship or transport combination in ERI. There are different codes for inland and ocean going vessels, single ships and transport combinations.

The “UN Rec 28” based Ship type table also includes a link (backward compatibility only and will be deleted in the future) to the old (2 digit) DVK code (Netherlands) based on the DVK code list from RWS-AVV.

Ship type code is also used (besides electronic reporting) when calculating the cones, to identify the type of ship / transport (motor freighter versus a motor tanker).

### Standard used

Ship type codes are based on the UN Recommendation 28, ECE/Trade/276; 2001/23. This recommendation establishes a common code list for the identification of the type of means of transport (in this case inland vessels and transport combinations). It has a particular relevance to transport organisations and providers, Customs and other authorities, statistical offices, forwarders, shippers, consignees and other parties concerned with transport.

Within ERI messages a subset of UN Rec 28 is used. In the ERI expert group it has been decided that the actual (operational) subset of shiptype codes to use is based on the specification in the COMMISSION REGULATION (EU) No 164/2010 (see ref [1]), including some (operational) additions. The complete (applicable) list of (operational) ERI shiptypes to be used is published in the ERDMS.

See also:

http://www.unece.org/cefact

http://live.unece.org/cefact/recommendations/rec_index.html (see UN rec 28)

http://eur-lex.europa.eu/ (EU 164/2010 regulation, see English section)

http:/risdatamanagement.ris.eu (ERDMS)

More information:

- Ship type UN Rec 28: www.unece.org/cefact/recommendations/rec_index.html

- Directive EU 164/2010 regarding electronic reporting: http://eur-lex.europa.eu/LexUriServ/LexUriServ.do?uri=CELEX:32010R0164:EN:NOT

### Encoding guideline Ship type

Usually subtypes are defined based on the generic type.

Encoding example:

8020 = Motor tanker

8021 = Motor tanker, liquid cargo, type N

8022 = Motor tanker, liquid cargo, type C

See UN Rec 28 standard and Regulation EU no 164/2010.

### Data example Ship type

The 4-digit code is the unique key. This code is built up in accordance with the UN Rec 28, at which the 1st position indicates the mode of transport (8=Inland Shipping, 1=Seagoing, recreation). The 2nd, 3rd and 4th positions indicate the type and sub type of the means of transport.

Ship types for individual hulls are:

Example:

| Code | Description |
| --- | --- |
| 8010 | MOTOR FREIGHTER |
| 8020 | MOTOR TANKER |
| 8030 | CONTAINER VESSEL |

Type codes for transport combinations are:

Example:

| Code | Description |
| --- | --- |
| 8130 | COUPLED TUG FREIGHTERS |
| 8140 | COUPLED TUG FREIGHTER/TANKER, 1TS |
| 8290 | PUSHTOW, > 8 CARGO BARGES |

## Container type (ISO 6346)

### Description Container type

The ERI Container types are identified using a 4 letter code based on the ISO 6346:1995 standard. In 2012 an update was published (ISO 6346:1995 Amendment 3, 2012(E)), which updated the meaning of the 3rd and 4th letter. Also the Tank container type “T” has been replaced by “K” and “N”. Below you will find both publications (the original version and the actuel amendment 3 version).

The ISO 6346 container type (new style since 1996) is built up of 4 characters:

- The 1st character refers to the code for the length (in feet and or inches).

- The 2nd character refers to the code for the width and height (in feet, inches).

- And the 3rd and 4th character refer to the type of container (general cargo, tank etc.).

The container size type codes are displayed on the containers and are used in the electronic reporting whenever available. Container type codes play an important role at calculating the cones in case of a dangerous goods transport in a container.

In the ERI list some additional (older, none ISO 6346) container type (size type) codes are also available (backward compatibility). They can be recognised by the fact that the last two digits are numbers (instead of G0, T0 etc., see 1.8.1.2).

#### ISO 6346 Size type code

The table below describes the actual ISO 6346 container type code standard:

1st character:

| Length (mm) | Feet, inches | 1st Code |
| --- | --- | --- |
| 2991 | 10 | 1 |
| 6068 | 20 | 2 |
| 9125 | 30 | 3 |
| 12192 | 40 | 4 |
| 7150 | - | A |
| 7315 | 24 | B |
| 7430 | 24,6 | C |
| 7450 | - | D |
| 7820 | - | E |
| 8100 | - | F |
| 12500 | 41 | G |
| 13106 | 43 | H |
| 13600 | - | K |
| 13716 | 45 | L |
| 14630 | 48 | M |
| 14935 | 49 | N |
| 16154 | - | P |

2nd character:

|  |  |  | 2nd Code characters: |  |  |
| --- | --- | --- | --- | --- | --- |
| Height |  |  | Width: |  |  |
| mm | Feet, inches |  | 2438mm (8ft) | >2438mm en <2500mm | >2500mm (8,2 ft) |
| 2438 | 8 |  | 0 (zero) |  |  |
| 2591 | 8,6 |  | 2 | C | L |
| 2743 | 9 |  | 4 | D | M |
| 2895 | 9,6 |  | 5 | E | N |
| >2895 | >9,6 |  | 6 | F | P |
| 1295 | 4,3 |  | 8 | - | - |
| =<1219 | <=4 |  | 9 | - | - |

3rd and 4th characters (original older ISO6346-1995, see newer version below):

| Code | Type Designation (original older, replaced ISO 6346 standard) | Group code | Main Characteristics (original older, replaced ISO 6346 standard) | Detailed Type Code (3rd, 4th) |
| --- | --- | --- | --- | --- |
| G | General purpose container without ventilation |  |  |  |
|  |  | GP | Openings at one or both ends | G0 |
|  |  |  | Passive vents at upper part of cargo space | G1 |
|  |  |  | Opening(s) at one or both ends plus "full" opening(s) on one or both sides | G2 |
|  |  |  | Opening(s) at one or both ends plus "partial" opening(s) on one or both sides | G3 |
| V | General purpose container with ventilation |  |  |  |
|  |  | VH | Non-mechanical system, vents at lower and upper parts of cargo space | V0 |
|  |  |  | Mechanical ventilation system, located internally | V2 |
|  |  |  | Mechanical ventilation system, located externally | V4 |
| B | Dry bulk container |  |  |  |
|  | Non pressurized, box type | BU | Closed | B0 |
|  |  |  | Airtight | B1 |
|  | Pressurized | BK | Horizontal discharge, test pressure 150 kPa | B3 |
|  |  |  | Horizontal discharge, test pressure 265 kPa | B4 |
|  |  |  | Tipping discharge, test pressure 150 kPa | B5 |
|  |  |  | Tipping discharge, test pressure 265 kPa | B6 |
| S | Named cargo container |  |  |  |
|  |  | SN | Livestock carrier | S0 |
|  |  |  | Automobile carrier | S1 |
|  |  |  | Live fish carrier | S2 |
| R | Thermal container |  |  |  |
|  | Refrigerated | RE | Mechanically refrigerated | R0 |
|  | Refrigerated and heated | RT | Mechanically refrigerated and heated | R1 |
|  | Self-powered refrigerated/heated | RS | Mechanically refrigerated | R2 |
|  |  |  | Mechanically refrigerated and heated | R3 |
| H | Thermal container |  |  |  |
|  | Refrigerated and/or heated with removable equipment | HR | Refrigerated and/or heated with removable equipment located externally; heat transfer coefficient K=0.4 W/(m2.K) | H0 |
|  |  |  | Refrigerated and/or heated with removable equipment located internally | H1 |
|  |  |  | Refrigerated and/or heated with removable equipment located externally; heat transfer coefficient K=0.7 W/(m2.K) | H2 |
|  | Insulated | HI | Insulated; heat transfer coefficient K=0.4 W/(m2.K) | H5 |
|  |  |  | Insulated; heat transfer coefficient K=0.7 W/(m2.K) | H6 |
| U | Open-top containers |  |  |  |
|  |  | UT | Opening(s) at one or both ends | U0 |
|  |  |  | Opening(s) at one or both ends, plus removable top member(s) in end frame(s) | U1 |
|  |  |  | Opening(s) at one or both ends, plus opening(s) on one or both sides | U2 |
|  |  |  | Opening(s) at one or both ends, plus opening(s) on one or both sides plus removable top member(s) in end frame(s) | U3 |
|  |  |  | Opening(s) at one or both ends, plus partial opening on one side and full opening on the other side | U4 |
|  |  |  | Complete, fixed side and end walls (no doors) | U5 |
| P | Platform (container) |  |  |  |
|  |  | PL | Platform (container) | P0 |
|  | Platform based containers with incomplete super-structure: |  |  |  |
|  | Fixed | PF | Two complete and fixed ends | P1 |
|  |  |  | Fixed posts, either free-standing or with removable top member | P2 |
|  | Folding (collapsible) | PC | Folding complete end structure | P3 |
|  |  |  | Folding posts, either free-standing or with removable top member | P4 |
|  | Platform-based containers with complete superstructure | PS | Open top, open ends (skeletal) | P5 |
| T | Tank container |  |  |  |
|  | For non-hazardous liquids | TN | Minimum pressure 45 kPa | T0 |
|  |  |  | Minimum pressure 150 kPa | T1 |
|  |  |  | Minimum pressure 265 kPa | T2 |
|  | For dangerous liquids | TD | Minimum pressure 150 kPa | T3 |
|  |  |  | Minimum pressure 265 kPa | T4 |
|  |  |  | Minimum pressure 400 kPa | T5 |
|  |  |  | Minimum pressure 600 kPa | T6 |
|  | For gases | TG | Minimum pressure 910 kPa | T7 |
|  |  |  | Minimum pressure 2 200 kPa | T8 |
|  |  |  | Minimum pressure (to be decided) | T9 |
| A | Air/Surface container |  |  |  |
|  |  | AS | Air/Surface (intermodal) container | A0 |

3rd and 4th characters (updated actual ISO 6346-1995 Amendment 3:2012(E)):

| Code | Type Designation | Group code | Main Characteristics | Detailed Type Code (3rd, 4th)a | Detailed Type Code (3rd, 4th)b |
| --- | --- | --- | --- | --- | --- |
| G | General purpose container without ventilation |  |  |  |  |
|  |  | GP | Openings at one or both ends | G0 | GA |
|  |  |  | Passive vents at upper part of cargo space | G1 | GB |
|  |  |  | Opening(s) at one or both ends plus "full" opening(s) on one or both sides | G2 | GD |
|  |  |  | Opening(s) at one or both ends plus "partial" opening(s) on one or both sides | G3 | GG |
|  |  |  | With bulk capabilities | G9 | GY |
| V | General purpose container with ventilation |  |  |  |  |
|  |  | VH | Non-mechanical system, vents at lower and upper parts of cargo space | V0 | VA |
|  |  |  | Mechanical ventilation system, located internally | V2 | VD |
|  |  |  | Mechanical ventilation system, located externally | V4 | VJ |
| B | Dry bulk containerd) |  |  |  |  |
|  | Non pressurized, box type | BU | Closed | B0 | BA |
|  |  |  | Airtight | B1 | BB |
|  |  |  | Rear discharge/cat flap type | B3 | BG |
|  |  |  | Rear discharge/full width opening | B4 | BJ |
|  |  |  | Rear discharge/full width fixed | B5 | BM |
|  |  |  | Front discharge/full width | B8 | BX |
|  |  |  | Side discharge | B9 | BY |
| S | Named cargo container |  |  |  |  |
|  |  | SN | Livestock carrier | S0 | SA |
|  |  |  | Automobile carrier | S1 | SB |
|  |  |  | Live fish carrier | S2 | SD |
|  |  |  | Generator | S4 | SJ |
| R | Thermal container |  |  |  |  |
|  | Refrigerated | RE | Mechanically refrigerated | R0 | RA |
|  | Refrigerated and heated | RT | Mechanically refrigerated and heated | R1 | RB |
|  | Self-powered refrigerated/heated | RS | Mechanically refrigerated | R2 | RD |
|  |  |  | Mechanically refrigerated and heated | R3 | RG |
| H | Thermal container |  |  |  |  |
|  | Refrigerated and/or heated with removable equipment | HR | Refrigerated and/or heated with removable equipment located externally; heat transfer coefficient K=0.4 W/(m2.K) | H0 | HA |
|  |  |  | Refrigerated and/or heated with removable equipment located internally | H1 | HB |
|  |  |  | Refrigerated and/or heated with removable equipment located externally; heat transfer coefficient K=0.7 W/(m2.K) | H2 | HD |
|  | Insulated | HI | Insulated; heat transfer coefficient K=0.4 W/(m2.K) | H5 | HM |
|  |  |  | Insulated; heat transfer coefficient K=0.7 W/(m2.K) | H6 | HV |
| U | Open-top containers |  |  |  |  |
|  |  | UT | Opening(s) at one or both ends | U0 | UA |
|  |  |  | Opening(s) at one or both ends, plus removable top member(s) in end frame(s) | U1 | UB |
|  |  |  | Opening(s) at one or both ends, plus opening(s) on one or both sides | U2 | UD |
|  |  |  | Opening(s) at one or both ends, plus opening(s) on one or both sides plus removable top member(s) in end frame(s) | U3 | UG |
|  |  |  | Opening(s) at one or both ends, plus partial opening on one side and full opening on the other side | U4 | UJ |
|  |  |  | Open topped container with removable hard top | U6 | UV |
| P | Platform (container) |  |  |  |  |
|  |  | PL | Platform (container) | P0 | PA |
|  | Platform based containers with incomplete super-structure: |  |  |  |  |
|  | -Fixed | PF | Two complete and fixed ends | P1 | PB |
|  |  |  | Fixed posts, either free-standing or with removable top member | P2 | PD |
|  | -Folding (collapsible) | PC | Folding complete end structure | P3 | PG |
|  |  |  | Folding posts, either free-standing or with removable top member | P4 | PJ |
|  | Platform-based containers with complete superstructure | PS | Open top, open ends (skeletal) | P5 | PM |
|  | Platform-based containers for named cargo |  | Ship’s gear carrier | P6 | PV |
|  |  |  | Car carrier | P7 | PW |
|  |  |  | Timber / Pipe carrier | P8 | PX |
|  |  |  | Coil carrier | P9 | PY |
| T | Tank containere) |  | <T0-T9 deleted> | - | - |
| K | Pressurized tank container (liquids and gases) |  |  |  |  |
|  |  | KL | Liquid tank non-regulated goods | K0 | KA |
|  |  |  | Liquid tank dangerous goods ≤ 2,65 barc pressure | K1 | KB |
|  |  |  | Liquid tank dangerous goods >2,65 barc and ≤ 10 barc pressure | K2 | KD |
|  |  |  | Liquid tank dangerous goods > 10 barc high pressure | K3 | KG |
|  |  |  | Liquid tank non regulated goods requiring power supply | K4 | KJ |
|  |  |  | Liquid tank for dangerous goods ≤ 10 barc requiring power supply | K5 | KM |
|  |  |  | Liquid tank for dangerous goods > 10 barc pressure requiring power supply | K6 | KV |
|  |  |  | Cryogenic tank | K7 | KW |
|  |  |  | Gas tank | K8 | KX |
| N | Pressurized and non-pressurized tank container (dry) |  |  |  |  |
|  | -Hopper | NH | Hopper type vertical discharge | N0 | NA |
|  |  |  | Hopper type rear discharge | N1 | NB |
|  | -Non pressurized | NN | Non pressurized rear discharge | N3 | NG |
|  |  |  | Non-pressurized side discharge | N4 | NJ |
|  |  |  | Non-pressurized tipping discharge | N5 | NM |
|  | -Pressurized | NP | Pressurized rear discharge | N7 | NW |
|  |  |  | Pressurized side discharge | N8 | NX |
|  |  |  | Pressurized tipping discharge | N9 | NL |
| A | Air/Surface container |  |  |  |  |
|  |  | AS | Air/Surface (intermodal) container | A0 | - |
| a) For containers designed and tested with full stacking (minimum superimposed mass of 192,000 kg) and racking (minimum transverse force of 150 kN) capabilities. Superimposed mass is as defined in ISO 1496-1:1990. b) This includes containers designed and tested with reduced stacking and/or racking capabilities, but not containers that are approved or operated with one door off or otherwise operated with a temporary reduced capability. c) 100 kPa = 1 bar = 105 Pa = 105 N/m2 = 14.5 lbf/in2 d) In Amendment 3 the Type Code B1-B5 are replaced with a different description (B6 definition is deleted). e) In Amendment 3 the Type Code definitions for T0-T9 (Tank containers) are deleted. |  |  |  |  |  |

The New ISO 6346 Amd 3 (updated) entries and or fields are marked with yellow.

More container information:

http://www.prefixlist.com/

http://shipping-container-info.com/size-type-info/shipping-container-size-type

#### Older Size type code

The older, none ISO 6346, container type can be recognised by the fact that the last two digits are numbers (instead of G0, T0 etc.).

Here is the description of the 'old' numeric coding. The total number of characters is 4 (four), which are all numeric. The first character depends on the length, the second character on the height. The 3rd and 4th character indicate the type.

The tables below describes the older (before ISO 6346) size type standard:

First size code character depends on container length, second on height:

| Size code | Length (ft) | Length (mm) |  | Height code | Height (ft) | Height in mm |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10’ | 2991 mm |  | 1 | 8’ | 2438 mm |
| 2 | 20’ | 6068 mm |  | 2 | 8’ | 2438 mm (tunnel) |
| 3 | 30’ | 9125 mm |  | 3 | 8’6 | 2591 mm |
| 4 | 40’ | 12192 mm |  | 4 | 8’6 | 2591 mm (tunnel) |
| 5 | unassigned | - |  | 5 | > 8’6 | > 2591 mm |
| 6 | unassigned | - |  | 6 | > 8’6 | > 2591 mm (tunnel) |
| 7 | unassigned | - |  | 7 | > 4' and <= 4'3 | > 1219 mm and <= 1295 mm |
| 8 | 35’ | 10668 mm |  | 8 | > 4' and <= 4'3 | > 1219 mm and <= 1295 mm (tun) |
| 9 | 40’ | 13716 mm |  | 9 | > 4'3 and <= 8' | > 1295mm and <= 2438mm |

The third and fourth character depends on container type:

| Type code | Type description (older none ISO size type) |
| --- | --- |
| 00 | DV General purpose containers, Opening(s) at one or both ends |
| 01 | DV General purpose containers, Opening(s) at one or both ends, plus 'full' opening(s) on one or both sides |
| 02 | DV General purpose containers, Opening(s) at one or both ends, plus 'partial' opening(s) on one or both sides |
| 03 | DV General purpose containers, Opening(s) at one or both ends, plus opening roof |
| 04 | DV General purpose containers, Opening(s) at one or both ends, plus opening roof and opening(s) at one or both sides |
| 05 | DV General purpose containers, Spare |
| 06 | General purpose containers, Spare |
| 07 | General purpose containers, Spare |
| 08 | General purpose containers, Spare |
| 09 | General purpose containers, Spare |
| 10 | DV Closed containers vented, Passive vents at upper part of cargo space. Total vent cross-section area < 25cm2/m of nominal length |
| 11 | DV Closed containers vented, Passive vents at upper part of cargo space. Total vent cross-section area >= 25cm2/m of nominal length |
| 12 | VE Closed containers vented, Vented, Spare |
| 13 | VE Closed containers Ventilated, Non mechanical system, vents at lower and upper parts of cargo space |
| 14 | DV Closed containers Ventilated, Spare |
| 15 | DV Closed containers Ventilated, Mechanical ventilation system, located internally |
| 16 | DV Closed containers Ventilated, Spare |
| 17 | Closed containers Ventilated, Mechanical ventilation system, located externally |
| 18 | DV Closed containers Ventilated, Spare |
| 19 | DV Closed containers Ventilated, Spare |
| 20 | I* Dry bulk containers, Non-pressurized, Box type, Closed |
| 21 | I* Dry bulk containers, Non-pressurized, Box type, Vented |
| 22 | I* Dry bulk containers, Non-pressurized, Box type, Ventilated |
| 23 | Dry bulk containers, Non-pressurized, Box type, Airtight |
| 24 | Dry bulk containers, Non-pressurized, Box type, Spare |
| 25 | Named cargo containers, Livestock carrier |
| 26 | Named cargo containers, Automobile carrier |
| 27 | Named cargo containers, Spare |
| 28 | Named cargo containers, Spare |
| 29 | Named cargo containers, Spare |
| 30 | R* Thermal containers, Refrigerated, Expendable refrigant |
| 31 | R* Thermal containers, Mechanically refrigerated |
| 32 | R* Thermal containers, Refrigerated and heated |
| 33 | Thermal containers, Heated |
| 34 | Thermal containers, Heated |
| 35 | Thermal containers, Heated |
| 36 | Thermal containers, Self powered mechanically refrigerated |
| 37 | R* Thermal containers, Self powered refrigerated and heated |
| 38 | Thermal containers, Self powered heated |
| 39 | Thermal containers, Self powered refrigerated/heated, Spare |
| 40 | I* Thermal containers, Refrigerated and/or heated with removable equipment located externally |
| 41 | I* Thermal containers, Refrigerated and/or heated with removable equipment located internally |
| 42 | I* Thermal containers, Refrigerated and/or heated with removable equipment located externally |
| 43 | Thermal containers, Refrigerated and/or heated with removable equipment, Spare |
| 44 | Thermal containers, Refrigerated and/or heated with removable equipment, Spare |
| 45 | Thermal containers, Insulated |
| 46 | Thermal containers, Insulated |
| 47 | Thermal containers, Insulated, Spare |
| 48 | Thermal containers, Insulated, Spare |
| 49 | Thermal containers, Insulated, Spare |
| 50 | OT Open top containers, Opening(s) at one or both ends |
| 51 | OT Open top containers, Opening(s) at one or both ends plus removable top member(s) in end frame(s) |
| 52 | OT Open top containers, Opening(s) at one or both ends plus opening(s) on one or both sides |
| 53 | OT Open top containers, Opening(s) at one or both ends plus opening(s) on one or both sides plus removable top member(s) in end frame(s) |
| 54 | OT Open top containers, Opening(s) at one or both ends plus 'partial' opening on one side and 'full' opening on the other side |
| 55 | OT Open top containers, Spare |
| 56 | Open top containers, Spare |
| 57 | Open top containers, Spare |
| 58 | Open top containers, Spare |
| 59 | HT Open top containers, Spare |
| 60 | FB Platform (container) |
| 61 | FL Platform based containers with incomplete superstructure with complete and fixed ends |
| 62 | FL Platform based containers with incomplete superstructure with fixed free-standing posts |
| 63 | FL Platform based containers with incomplete superstructure with folding complete end structure |
| 64 | FL Platform based containers with incomplete superstructure with folding free-standing posts |
| 65 | OS Platform based containers with complete superstructure with roof |
| 66 | OS Platform based containers with complete superstructure with open top |
| 67 | OS Platform based containers with complete superstructure with open top, open ends (skeletal) |
| 68 | Platform based containers with complete superstructure, Spare |
| 69 | Platform based containers with complete superstructure, Spare |
| 70 | T* Tank containers, Non dangerous liquids, test pressure 45 kPa |
| 71 | T* Tank containers, Non dangerous liquids, test pressure 150 kPa |
| 72 | T* Tank containers, Non dangerous liquids, test pressure 265 kPa |
| 73 | T* Tank containers, Dangerous liquids, test pressure 150 kPa |
| 74 | T* Tank containers, Dangerous liquids, test pressure 265 kPa |
| 75 | T* Tank containers, Dangerous liquids, test pressure 400 kPa |
| 76 | T* Tank containers, Dangerous liquids, test pressure 600 kPa |
| 77 | T* Tank containers, Dangerous gases, test pressure 1050 kPa |
| 78 | T* Tank containers, Dangerous gases, test pressure 2200 kPa |
| 79 | T* Tank containers, Dangerous gases, test pressure unassigned |
| 80 | BK Dry Bulk container, Non pressurized, hopper type, Closed |
| 81 | BK Dry Bulk container, Non pressurized, hopper type, Vented |
| 82 | BK Dry Bulk container, Non pressurized, hopper type, Ventilated |
| 83 | Dry Bulk container, Non pressurized, hopper type, Airtight |
| 84 | Dry Bulk container, Non pressurized, hopper type, Spare |
| 85 | Dry Bulk container, Pressurized, Horizontal discharge, test pressure 150 kPa |
| 86 | FL Dry Bulk container, Pressurized, Horizontal discharge, test pressure 265 kPa |
| 87 | Dry Bulk container, Pressurized, Tipping discharge, test pressure 150 kPa |
| 88 | Dry Bulk container, Pressurized, Tipping discharge, test pressure 265 |
| 89 | Dry Bulk container, Pressurized, Spare |
| 90 | Air/surface containers, Spare |
| 91 | Air/surface containers, Spare |
| 92 | Air/surface containers, Spare |
| 93 | Air/surface containers, Spare |
| 94 | Air/surface containers, Spare |
| 95 | Air/surface containers, Spare |
| 96 | Air/surface containers, Spare |
| 97 | Air/surface containers, Spare |
| 98 | Air/surface containers, Spare |
| 99 | CO Air/surface containers, Spare |

### Standard used

The container types are based on the ISO 6346 1995 standard (new style since 1996) and updated with Amendment 3. Some common used older types (before ISO 6346, such as 2200, 4200 etc.) are also still included.

The code consists of 4 characters specifying the length, height and type of the container (see paragraph 1.8.1).

In 2012 an update was published (ISO 6346:1995 Amendment 3, 2012(E)), which updated the meaning of the 3rd and 4th letter. The “T” group was deleted/replaced with new “K” and “N” groups, the “B” group descriptions were changed and finally a second type code for ‘reduced stackable’ containers was introduced.

See also:

http://www.iso.org

http://shipping-container-info.com/

More information:

- ISO codes: www.iso.org

- Container ISO 6346 codes: http://en.wikipedia.org/wiki/ISO_6346

### Encoding guideline Container type

See paragraph 1.8.1 (description of container type codes)

The ISO 6346/Am3 contains the following definitions:

- 17 different length types.

- 15 different width & height types.

- ~161 different subtypes:

  - 23 group-code subtypes

  - 75 (basic) subtypes (detail code column #1)

  - 63 subtypes for reduced stacking (detail code column #2)

Summary of the encoded (applicable) container types (based on ISO 6346/Am3):

- Common used older types (before ISO 6346) are still included for compatibility reasons (such as 2200, 4200 etc.).

- Included older ISO 6346 subtypes T0-T9 (backwards compatibility)

- Deleted older ISO 6346 subtypes B1-B6 (they are replaced by a completely new description in Amendment 3).

- Included ISO 6346/Am3, 10ft subset of normal width & height sizes: 0, 2, 5, 8, 9, C, E, F, L, N and P  (for all subtypes, excl. half height tank containers).

- Included ISO 6346/Am3, 20, 30, 40, 45ft all width & height sizes (for all subtypes, excl. half height tank containers).

- Included ISO 6346/Am3, deviating sizes 23, 25, 26ft Tank containers, all width & height sizes, excl. half height tank containers (for all tank container subtypes T, K, N).

- Included ISO 6346/Am3, deviating sizes: 48, 53ft (sea) containers, a subset of normal width & height sizes: 0, 2, 5, 6, 8, C, E, F, L, N and P  (for all basic subtypes, excl. reduced stacking types).

### Data example Container type

The 4-letter container type code (ISO 6364) forms the unique key in the container type table.

Example for container type:

| Code | Description |
| --- | --- |
| 2000 | 20FT GEN. PURP. CONT. NON-VENTED (8) |
| 20G0 | 20' General purpose non -vented container |
| 40T0 | 40' Tank Container (Minimum pressure 45 kPa) |
| L2G1 | 45FT GEN. PURP. CONT. PASSIVE VENTS |

## Inner Packaging type (UN Rec 21)

### Description Packaging type

The ERI Package type is a 2 letter code based on the Un Recommendation 21 standard. A numeric code system to describe the appearance of packed goods (inner packing types and materials) as presented for transport to facilitate identification, recording, handling, and establishing handling tariffs.

### Standard used

The (inner) packaging type consists of a 2-letter code and is based on the UN/ECE Trade Facilitation Recommendation 21.

See also:

http://www.unece.org/cefact

http://live.unece.org/cefact/recommendations/rec_index.html (see UN rec 21)

### Encoding guideline Packaging type

See paragraph 1.9.2 (description of code)

### Data example Packaging type

The 2-letter inner packaging code forms the unique key in the packaging type table.

Example:

| Code | Description |
| --- | --- |
| AE | Aerosol |
| AM | Ampoule, non-protected |
| AP | Ampoule, protected |
| NE | UNPACKED/UNPACKAGED |
| 4A | BOX, STEEL |
| 4B | BOX, ALUMINIUM |

Bijlage A -  Data maintenance and tooling

The maintenance and distribution of the mentioned Reference Data is facilitated by using a Website and special tooling (EU Reference Data Management Service - Tool = ERDMS-Tool).

Using this ERDMS tool so called Master dumps of the data can be requested and mutations (authorised users) can be specified.

The ERDMS can be accessed through this URL: http://risdatamanagement.ris.eu.

The documentation and a master dump of the data can be accessed without an account: http://risdatamanagement.ris.eu/reftool/masterdump/

For the full use (country Data manager) of the ERDMS – Tool you need an account.

Bijlage B -  Reference Data update procedure (overview)

This section provides an global overview of the update procedure for the reference data as maintained and supported by the ERDMS Tool (for details about the EU maintenance procedures see also ref [3])

The schematic diagram below gives an overview of the general Update procedure regarding Electronic Reporting International (ERI) and especially for the so called ERI applications:

Figure 4: Reference data Update procedure

Changes, which include additions as well as deletions, can come from various sources, as shown in the figure.

The (change) information is supplied using the web forms of the ERDMS Tool, an Excel Template or by the Web service (connected systems only).

The Data Manager assesses and validates these changes.  There might be some experts check and  the details of this data will be completed and will be checked where required with the involved international standardisation organisations. Any required deviations or remarks to the proposed changes are communicated with the originator.

Finally, the Data manager accepts the changes and he will publish the (changed) data (on the European level). Changes are collected by the Data Manager after which an Update is produced and sent to the BICS/ERI-Net users and to other relevant partners and systems, such as IVS90, MIB etc by the BICS Helpdesk(a NL specific task).

Partners can also download the actual Master tables from the ERDMS Tool. The (EU) partners and database managers of the related systems using the mentioned reference data are responsible to process and incorporate the published Updates (actual reference data) in their systems.

In summary (see figure), the change procedure consists of the following procedure:

- Data Manager receives requests for changes to the code tables.

- Data Manager checks and completes (validates) the data with the help of experts.

- Data Manager incorporates changes in Master Tables (process changes).

- Data Manager sends feedback on modifications to originator.

- Data manager publishes the updates (changes) on the European level.

Updates are sent to the Helpdesks, responsible members (per country) and all relevant partners (IVS90, MIB etc), so they can update their systems (NL specific task).

- Helpdesk sends the Update via the mailing list to fairway users (skippers) and other interested parties and archives the Update (in order to be able to be sent it again). At this moment this is a NL specific task.

The EU Data manager can be contacted via the web site:

http://risdatamanagement.ris.eu

The ERDMS covers the reference data used by ERI (Electronic Reporting International) and NtS (Notices to Skippers). Because the NtS codes are more or less static (read-only in ERDMS) and already described in the Notices to Skippers standard, we will not describe/explain them any further in this document.

ADN(R) recently shortened to ADN, because it’s not only applicable on the Rhine anymore, but now also applies to several other important EU waterways.

The Fallback mechanism: a mechanism where longer codes can be stripped down to an existing basic code (in this case for ERI locations the UN-Locode). So for every ERI location code a basic code (some what duplicate and obsolete for normal usage) will/must be available!

Where (detail) characteristics of the container type are unspecified or unknown the type can be identified by its group code.